TCP(4)              Xinu Programmer's Manual               TCP(4)

NAME

tcp - interface to reliable 2-way Internet stream communica-

tion

SYNOPSIS

#include <network.h>

#include <tcb.h>

#include <tcpstat.h>

read(device, buffer, length)

write(device, buffer, length)

putc(device, char)

getc(char)

open(device, name, lport)

close(device)

control(device, function, arg1, arg2)

DESCRIPTION

The _#t_#c_#p_#m and _#t_#c_#p devices provide an interface to the Inter-

net TCP protocol.  Programs use the master device to allo-

cate a new TCP connection endpoint or to affect global TCP

configuration parameters for subsequent connections.  The

slave devices are particular instances of TCP servers or

active TCP connections.

The TCP devices support the following operations:

open(device, name, lport)

Allocate a new TCP connection endpoint.  Name is a

string that gives the IP address and TCP port number of

the foreign endpoint in the form _#i_#1._#i_#2._#i_#3._#i_#4:_#t for

active opens, or the manifest constant ANYFPORT for

passive opens (servers).  Lport specifies the local

port. If lport is ANYLPORT, Xinu will pick a port to

use.

control(device, function, arg1)

The master device supports only the TCPC_LISTENQ _#c_#o_#n_#-

_#t_#r_#o_#l function.  It sets the default listen queue size

for all subsequent passive opens to the integer value

arg1. The default listen queue size at boot is 5.

For all other TCP devices, the following control functions

are defined:

TCPC_ACCEPT

For servers, accept any incoming connection attempts

for a port.

TCPC_LISTENQ

Set the listen queue length to the integer value in

_#a_#r_#g_#1.

Version 6b              Printed 5/23/103                        1

TCP(4)              Xinu Programmer's Manual               TCP(4)

TCPC_STATUS

Return the status of a TCP connection. _#A_#r_#g_#1 is a

pointer to struct tcpstat which on successful return

will contain the relevant statistics for the connec-

tion.

TCPC_SOPT/TCPC_COPT

Set or clear TCP user options. The current options are

TCBF_DELACK (do delayed ACK's) and TCBF_BUFFER (buffer

TCP input until the full count on a read is available).

_#A_#r_#g_#1 contains the flag value(s) to set or clear.

TCPC_SENDURG

Send urgent data. _#A_#r_#g_#1 is a pointer to a buffer of

urgent data and _#a_#r_#g_#2 is the size (in bytes) of the data

to send.

read(device, buffer, length)

Read length characters into buffer from a connected TCP

device.  The return value is the count of characters,

if non-negative. Negative values indicate various error

and exceptional conditions defined as manifest con-

stants TCPE_* in file ``tcb.h''.

write(device, buffer, length)

Write length bytes from buffer to the connected TCP

device. The return value is the number of characters

written, if non-negative, and an error or exceptional

condition, otherwise.

close(device)

Close a TCP device. For connected devices, the call

blocks until all pending data has been transmitted and

acknowledged. A nonzero return value indicates that

some data may have been lost.

SEE ALSO

udp(4)

Version 6b              Printed 5/23/103                        2